CISCO SYSTEMS

Software Development Engineer (SDE)

Interview Preparation Guide

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

COMPUTER NETWORKS — COMPLETE DEEP DIVE

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

📌 What's Covered ✔ Fundamentals of Data Communication & Networking ✔ OSI Model & TCP/IP Protocol Suite ✔ Data & Signals — Analog, Digital, Bandwidth, Nyquist & Shannon ✔ Modulation & Multiplexing (ASK/FSK/PSK, FDM/TDM/WDM) ✔ Guided & Unguided Transmission Media ✔ Data Link Layer — Error Detection/Correction, Framing, Protocols ✔ Network Layer — IPv4, IPv6, NAT, Routing Algorithms ✔ Transport Layer — TCP, UDP, Congestion Control, Traffic Management

Prepared by: Senior Technical HR | Cisco Systems

Confidential — For Interview Preparation Only | 2025-2026

# Quick Reference — Key Formulas & Facts

Bookmark this page. These are the most commonly tested formulas at Cisco CN interviews.

| Topic | Formula / Fact | Example |
| --- | --- | --- |
| Nyquist Bit Rate | 2 × BW × log₂(L) | BW=3kHz, L=8 → 18 Kbps |
| Shannon Capacity | BW × log₂(1 + SNR) | BW=3kHz, SNR=1000 → 30 Kbps |
| Hamming Redundancy | 2^r ≥ m + r + 1 | m=7 → r=4 (total 11 bits) |
| Mesh Links | n(n-1)/2 | n=5 → 10 links |
| ALOHA Efficiency | Pure=18.4% ¦ Slotted=36.8% | 1/(2e) vs 1/e |
| GBN Window | ≤ 2^n - 1 | n=3 → max 7 |
| SR Window | ≤ 2^(n-1) | n=3 → max 4 |
| Subnets from borrowed bits | 2^b subnets, 2^h - 2 hosts | /24 → /26: 4 subnets, 62 hosts |
| SNR (dB) | 10 × log₁₀(S/N) | S/N=1000 → 30 dB |
| IPv6 Address Space | 2^128 = 3.4 × 10^38 | 128-bit, hex, 8 groups of 4 |
| TCP Handshake | SYN → SYN-ACK → ACK | 3 packets to establish |
| TCP Teardown | FIN → ACK → FIN → ACK | 4 packets; TIME_WAIT = 2×MSL |

# 1. Fundamentals of Data Communication & Networking

| Q1: What are the five components of data communication? Explain each briefly. |  |
| --- | --- |
| ✅ Answer: The five essential components are: - Message – The actual data/information to be communicated (text, number, picture, audio, video). - Sender – The device that sends the message (computer, phone, camera). - Receiver – The device that receives the message. - Transmission Medium – Physical path through which the message travels (cable, air, fiber). - Protocol – A set of rules governing data communication (TCP/IP, HTTP, etc.). 📝 Note: Cisco interviewers often ask you to map these to a real scenario — e.g., WhatsApp call: your phone=sender, friend's phone=receiver, internet=medium, WebRTC=protocol. 💡 TIP: Remember with mnemonic: 'My Smart Robot Talks Properly' (Message, Sender, Receiver, Transmission medium, Protocol). 🔗 ANALOGY: Like sending a letter: the letter=message, you=sender, post office=medium, your friend=receiver, postal rules=protocol. |  |
| Q2: Differentiate between Simplex, Half-Duplex, and Full-Duplex communication with real-world examples. |  |
| ✅ Answer: - Simplex: Data flows in ONE direction only. - Example: TV broadcast, keyboard-to-CPU. - No feedback possible from receiver. - Half-Duplex: Data flows in BOTH directions but NOT simultaneously. - Example: Walkie-talkie (say "over" to switch), older Ethernet (CSMA/CD). - One party waits while other transmits. - Full-Duplex: Data flows in BOTH directions SIMULTANEOUSLY. - Example: Telephone call, modern Ethernet switch, USB 3.0. - Most efficient; bandwidth is fully utilised in both directions. Key formula insight: Full-duplex effectively DOUBLES the bandwidth compared to half-duplex. 💡 TIP: At Cisco, relate half-duplex to CSMA/CD (Ethernet hubs) and full-duplex to Ethernet switches — this shows real-world mapping. |  |
| Q3: What is a network? What are the three key criteria for a good network? |  |
| ✅ Answer: A network is an interconnection of two or more devices that can exchange data. Three key criteria: - Performance: Measured by throughput (data delivered per unit time) and delay (latency). Higher throughput + lower latency = better performance. - Reliability: Measured by frequency of failure, recovery time, and redundancy. Expressed as 'five 9s' (99.999% uptime). - Security: Protection of data from unauthorized access, damage, or loss. Includes confidentiality, integrity, and availability (CIA triad). 🔗 ANALOGY: Think of a highway system: performance = cars/hour that pass, reliability = how rarely the road closes, security = toll booths and cameras preventing unauthorized vehicles. |  |
| Q4: Compare Mesh, Star, Bus, and Ring topologies. Which does Cisco recommend for enterprise networks? |  |
| ✅ Answer: - Mesh Topology: - Every device connects to every other device. - Connections needed: n(n-1)/2 links. - Pros: Most reliable, no single point of failure. - Cons: Expensive, complex cabling. - Use: WAN backbone, critical military networks. - Star Topology: - All devices connect to a central hub/switch. - Pros: Easy to add/remove nodes, fault isolation (one cable fails, only that node affected). - Cons: Single point of failure at the hub/switch. - Use: Most common LAN topology. Cisco enterprise switches use this. - Bus Topology: - All devices share a single backbone cable. - Pros: Simple, cheap. - Cons: Single point of failure (backbone), collision-prone, hard to troubleshoot. - Obsolete: Replaced by star topology. - Ring Topology: - Devices connected in a closed loop. - Token-passing (Token Ring) prevents collisions. - Pros: Equal access, predictable performance. - Cons: Single break disrupts entire ring (unless dual-ring like FDDI). - Use: SONET/SDH (dual-ring for fault tolerance). Cisco recommends: Extended Star (hierarchical star) for enterprise — core, distribution, and access layers. 💡 TIP: Remember the mesh formula: n(n-1)/2. For n=4 devices, you need 4×3/2 = 6 links. This is a classic exam/interview calculation. |  |
| Q5: Differentiate LAN, MAN, and WAN. What technology is typical for each? |  |
| ✅ Answer: - LAN (Local Area Network): - Size: Single building / campus (up to ~1 km). - Speed: 100 Mbps to 10+ Gbps. - Technology: Ethernet (802.3), Wi-Fi (802.11). - Owned privately; low latency. - MAN (Metropolitan Area Network): - Size: City-wide (up to ~50 km). - Speed: 10 Mbps to 1 Gbps. - Technology: Metro Ethernet, WiMAX, FDDI, DSL. - Often operated by ISPs for city connectivity. - WAN (Wide Area Network): - Size: Country/global (unlimited). - Speed: Variable (dial-up to 100 Gbps backbone). - Technology: MPLS, Frame Relay, ATM, leased lines, Internet. - The Internet is the largest WAN. 🔗 ANALOGY: LAN = streets in your colony. MAN = all roads in your city. WAN = national highway + international borders. |  |
| Q6: Compare Circuit Switching vs. Packet Switching. Why does the internet use Packet Switching? |  |
| ✅ Answer: - Circuit Switching: - Dedicated path established BEFORE communication. - Resources reserved end-to-end for entire session duration. - Guaranteed bandwidth, fixed delay, no congestion. - Inefficient: channel idle during silence (e.g., old PSTN phone calls). - Example: Traditional telephone network (PSTN). - Packet Switching: - Data divided into packets, each routed independently. - Packets may take different paths and arrive out of order. - Resources shared (statistical multiplexing). - Efficient: bandwidth used only when data sent. - Example: Internet (IP), Ethernet. Why Internet uses Packet Switching: - Better utilization of network resources. - More resilient: if one path fails, packets reroute. - Supports bursty traffic patterns typical of web/email/video. - Cost-effective: no dedicated circuit reservation. Downside of Packet Switching: Variable latency (jitter) — problematic for real-time voice/video. That's why VoIP uses QoS (Quality of Service) to prioritize packets. 💡 TIP: Cisco interviewers love this: 'Why does PSTN use circuit switching and internet use packet switching?' Know that VoIP (which Cisco pioneered) runs packet switching over IP but uses QoS to mimic circuit-switching guarantees. |  |

# 2. OSI Model & TCP/IP Protocol Suite

| Q1: Explain the 7 layers of the OSI model with functions and protocols at each layer. |  |
| --- | --- |
| ✅ Answer: Layer 7 – Application: User-facing services. HTTP, FTP, SMTP, DNS, SNMP. Layer 6 – Presentation: Data translation, encryption, compression. SSL/TLS, JPEG, MPEG, ASCII. Layer 5 – Session: Manage sessions (open/close/maintain connections). NetBIOS, RPC, PPTP. Layer 4 – Transport: End-to-end delivery, error control, flow control. TCP, UDP, SCTP. Layer 3 – Network: Logical addressing, routing across networks. IP, ICMP, ARP (debated), OSPF, BGP. Layer 2 – Data Link: Framing, MAC addressing, error detection. Ethernet, PPP, Wi-Fi (802.11), HDLC. Layer 1 – Physical: Bits transmission over physical medium. RS-232, Ethernet cables, fiber, radio waves. PDU (Protocol Data Unit) by layer: - Layer 7/6/5 → Data ¦ Layer 4 → Segment ¦ Layer 3 → Packet ¦ Layer 2 → Frame ¦ Layer 1 → Bits 📝 Note: OSI is a conceptual model — no protocol uses all 7 layers strictly. TCP/IP collapses it into 4. At Cisco, SDE interviews often ask OSI in context of where firewalls (L3-L4), switches (L2), routers (L3) operate. 💡 TIP: Mnemonic (top-down): 'All People Seem To Need Data Processing' (Application, Presentation, Session, Transport, Network, Data Link, Physical). Bottom-up: 'Please Do Not Throw Sausage Pizza Away'. |  |
| Q2: Compare OSI model vs. TCP/IP model. Which one is used in practice and why? |  |
| ✅ Answer: OSI (7 layers): Application, Presentation, Session, Transport, Network, Data Link, Physical. TCP/IP (4 layers): Application, Transport, Internet, Network Access. Mapping: - OSI L7+L6+L5 → TCP/IP Application layer - OSI L4 → TCP/IP Transport layer - OSI L3 → TCP/IP Internet layer - OSI L2+L1 → TCP/IP Network Access layer TCP/IP is used in practice because: - Developed before OSI; Internet was already built on it. - Simpler and practical — fewer, larger layers. - OSI was designed by committee and arrived late. - TCP/IP is an open standard, free to use. OSI is used as a reference model for troubleshooting and education. When a network engineer says 'L3 issue' they mean routing/IP problem — OSI provides common language even when TCP/IP is running. 🔗 ANALOGY: OSI = the detailed blueprint/theory. TCP/IP = the actual building constructed. Architects (educators) use OSI; engineers (practitioners) use TCP/IP. |  |
| Q3: What happens step by step when you type www.cisco.com in your browser? Map it to OSI layers. |  |
| ✅ Answer: This is a complete end-to-end flow: 1. [L7 App] Browser creates HTTP GET request for www.cisco.com. 2. [L7 App] DNS resolution: OS checks cache → router cache → ISP DNS → recursive lookup → returns IP (e.g., 72.163.4.161). 3. [L4 Transport] TCP 3-way handshake: SYN → SYN-ACK → ACK establishes connection to port 443 (HTTPS). 4. [L6 Presentation] TLS handshake: negotiate cipher suite, exchange certificates, establish encrypted session. 5. [L7 App] HTTP GET request sent over encrypted TLS connection. 6. [L4 Transport] TCP segments the request, adds port numbers (src: ephemeral, dst: 443). 7. [L3 Network] IP adds source and destination IP; routing via OSPF/BGP through routers. 8. [L2 Data Link] Ethernet frames with MAC addresses; ARP used if needed within LAN. 9. [L1 Physical] Electrical/optical signals travel over cable/fiber/wireless. 10. [Reverse] Cisco's server processes, responds; data travels back same path; browser renders page. 💡 TIP: Memorize this flow — it's one of the most common 'walk me through' interview questions at Cisco. Mention DNS, TCP handshake, TLS, ARP, routing — shows depth. |  |

# 3. Data & Signals — Analog, Digital, Bandwidth

| Q1: What is the difference between analog and digital signals? What is bandwidth in each context? |  |
| --- | --- |
| ✅ Answer: - Analog Signal: - Continuous signal; can take any value in a range. - Described by: amplitude, frequency, phase. - Bandwidth = range of frequencies (Hz). E.g., human voice: 300 Hz to 3400 Hz = 3100 Hz bandwidth. - Susceptible to noise; degrades over distance. - Digital Signal: - Discrete signal; only two levels: 0 and 1. - Described by: bit rate (bps), bit length. - Bandwidth (digital context) = maximum bit rate (bps) the channel can carry. - More noise-resistant; can be regenerated perfectly. - Periodic Signal: Repeats pattern over time period T. Frequency f = 1/T. - Non-Periodic Signal (Aperiodic): No repeating pattern. Most data signals are aperiodic. - Composite Signal: Real signals are made up of many frequencies (by Fourier's theorem). A single square wave = infinite sine wave harmonics. Key insight: Digital signals occupy INFINITE bandwidth theoretically (due to sharp edges). In practice, we limit the bandwidth which introduces some signal degradation. 🔗 ANALOGY: Analog = dimmer switch (infinite brightness levels). Digital = on/off light switch (only 2 states). |  |
| Q2: Explain Nyquist Bit Rate and Shannon Capacity. How do they differ? Give formulas and examples. |  |
| ✅ Answer: - Nyquist Bit Rate (Noiseless Channel): Formula: Bit Rate = 2 × Bandwidth × log₂(L) Where L = number of signal levels. Example: Bandwidth = 3000 Hz, L = 8 levels. Bit Rate = 2 × 3000 × log₂(8) = 2 × 3000 × 3 = 18,000 bps = 18 Kbps. Nyquist says: to perfectly reconstruct a signal, sample at 2× the highest frequency (Nyquist rate). - Shannon Capacity (Noisy Channel): Formula: Capacity = Bandwidth × log₂(1 + SNR) Where SNR = Signal-to-Noise Ratio (ratio, not dB). Example: Bandwidth = 3000 Hz, SNR = 35 dB. First convert: SNR (linear) = 10^(35/10) ≈ 3162. Capacity = 3000 × log₂(1 + 3162) ≈ 3000 × 11.63 ≈ 34,881 bps ≈ 34.9 Kbps. Key Differences: - Nyquist: determines max bit rate for NOISELESS channel; depends on signal levels (L). - Shannon: sets THEORETICAL UPPER LIMIT for noisy channel; no amount of signal levels can exceed it. - Shannon sets the ceiling; Nyquist tells you how many levels you need to approach it. 💡 TIP: In interviews: 'What's the max data rate of a telephone channel?' — Answer with Shannon (noisy real world). If they say noiseless, use Nyquist. Always check: Shannon gives absolute ceiling. |  |
| Q3: What are Attenuation, Distortion, and Noise? How are they mitigated in real networks? |  |
| ✅ Answer: Three transmission impairments: - Attenuation: Signal loses energy (gets weaker) as it travels. - Measured in decibels (dB). Higher frequency = more attenuation. - Mitigation: Amplifiers (analog) or Repeaters/Regenerators (digital). - Example: Why Wi-Fi signal weakens as you move away from router. - Distortion: Signal changes shape due to different frequencies travelling at different speeds. - Each frequency component arrives at different times → waveform distorted at receiver. - Mitigation: Equalizers that compensate for frequency-dependent delays. - Example: Long copper cable distorts high-frequency components of voice signal. - Noise: Unwanted energy added to the signal. - Types: Thermal noise (always present due to temperature), Induced noise (from motors/appliances), Crosstalk (signal from adjacent wire), Impulse noise (voltage spikes, lightning). - Mitigation: Shielding (STP cable), filtering, error correction codes, better SNR. The SNR (Signal-to-Noise Ratio) in dB = 10 × log₁₀(Signal Power / Noise Power). Higher SNR = cleaner signal = higher capacity (see Shannon). 🔗 ANALOGY: Attenuation = shouting across a large room (voice gets weaker). Distortion = voice echoing and reverberating (shape changes). Noise = construction noise drowning your voice. |  |
| Q4: What is the difference between bit rate and baud rate? Why does this matter? |  |
| ✅ Answer: - Bit Rate: Number of BITS transmitted per second (bps). - Baud Rate (Symbol Rate): Number of SIGNAL CHANGES (symbols) per second. Relationship: Bit Rate = Baud Rate × bits per symbol = Baud Rate × log₂(L) Example: If L=4 signal levels (2 bits per symbol), Baud Rate = 1000 baud. Then Bit Rate = 1000 × log₂(4) = 1000 × 2 = 2000 bps. Why it matters: - Physical medium limits BAUD RATE (how fast you can change signals). - By encoding more bits per symbol, you increase bit rate without increasing baud rate. - This is how QAM (Quadrature Amplitude Modulation) achieves high speeds in cable modems and Wi-Fi. - Example: 256-QAM uses 8 bits per symbol. At 1000 baud → 8000 bps. 💡 TIP: Cisco Wi-Fi/cable modem products heavily use QAM. Knowing baud vs bit rate explains why 802.11ac/Wi-Fi 5 achieves Gbps speeds — they use 256-QAM with MIMO. |  |

# 4. Digital-to-Analog & Analog-to-Analog Conversion, Multiplexing

| Q1: Explain ASK, FSK, and PSK with their characteristics and use cases. |  |
| --- | --- |
| ✅ Answer: These are Digital-to-Analog modulation techniques for sending digital data over analog channels: - ASK (Amplitude Shift Keying): - Changes the AMPLITUDE of carrier wave to represent 0s and 1s. - Example: OOK (On-Off Keying) — signal ON = 1, signal OFF = 0. - Pros: Simple, low bandwidth. - Cons: Very susceptible to noise (noise affects amplitude directly). - Use: Optical fiber transmission (light on/off), RFID, simple remote controls. - FSK (Frequency Shift Keying): - Changes the FREQUENCY of carrier wave. - 0 = lower frequency, 1 = higher frequency. - Pros: Less susceptible to amplitude noise than ASK. - Cons: Requires more bandwidth. - Use: Old dial-up modems (Bell 103: 1070 Hz=0, 1270 Hz=1), IoT sensors, telemetry. - PSK (Phase Shift Keying): - Changes the PHASE of carrier wave. - BPSK (Binary PSK): 0° = 0, 180° = 1. - QPSK: 4 phases, 2 bits per symbol. - Pros: More noise-immune than ASK; more bandwidth-efficient than FSK. - Cons: Requires phase synchronization between sender and receiver. - Use: Wi-Fi (802.11), satellite comms, cable modems, 3G/4G/5G. Advanced: QAM = ASK + PSK combined. 64-QAM, 256-QAM used in LTE, cable TV, Wi-Fi. 💡 TIP: Remember: PSK is the most efficient among the three for wireless. QAM (combination) is what Cisco Wi-Fi products actually use for maximum throughput. |  |
| Q2: Explain AM, FM, and PM modulation. How do they differ from digital modulation? |  |
| ✅ Answer: These are Analog-to-Analog conversion techniques (analog data → analog signal at different frequency): - AM (Amplitude Modulation): - Information encoded in amplitude changes of carrier. - Carrier frequency stays constant; amplitude varies with signal. - Bandwidth = 2 × signal bandwidth. AM radio uses ~10 kHz per station. - Pros: Simple receivers, long range. - Cons: Very noise-susceptible (thunderstorms cause static). - Use: AM radio (535 kHz – 1705 kHz), aircraft communication. - FM (Frequency Modulation): - Information encoded in frequency deviations of carrier. - Amplitude stays constant; frequency varies with signal. - Bandwidth larger than AM (Carson's rule: BW ≈ 2(Δf + fm)). - Pros: Much better noise immunity, higher audio quality. - Cons: More bandwidth, more complex circuits. - Use: FM radio (88–108 MHz), VHF two-way radios, Bluetooth (GFSK variant). - PM (Phase Modulation): - Information encoded in phase changes of carrier. - Similar to FM but derivative of message changes frequency. - Use: Data modems, satellite communications (underpins PSK). Key difference from digital modulation: These carry ANALOG information (continuous audio) on an analog carrier. Digital modulation (ASK/FSK/PSK) carries DISCRETE digital bits. 🔗 ANALOGY: AM = changing volume to convey a message. FM = changing pitch/speed of speaking. PM = changing the timing of syllables. |  |
| Q3: Explain FDM, TDM, and WDM. How does each maximize channel utilization? |  |
| ✅ Answer: Multiplexing = combining multiple signals over one physical link. - FDM (Frequency Division Multiplexing): - Each user gets a DIFFERENT FREQUENCY BAND simultaneously. - Guard bands prevent interference between channels. - Analog technique; works continuously. - Example: Cable TV (each channel at different frequency), radio stations (88-108 MHz each at different freq), ADSL (voice at 0-4 kHz, data at 25 kHz-1.1 MHz). - Pros: Simple, no synchronization needed, each user has dedicated bandwidth. - Cons: Guard bands waste spectrum; rigid allocation. - TDM (Time Division Multiplexing): - Each user gets the FULL BANDWIDTH for a SHORT TIME SLOT. - Slots rotate cyclically: user1 → user2 → user3 → user1... - Digital technique; requires synchronization. - Types: Synchronous TDM (fixed slots, even if empty) and Statistical TDM (flexible, no wasted slots). - Example: T1/DS1 line (24 channels × 64 Kbps = 1.544 Mbps), GSM digital cellular. - Pros: Efficient for digital data; easy to add/remove users. - Cons: Synchronous TDM wastes capacity with idle slots. - WDM (Wavelength Division Multiplexing): - Each user gets a DIFFERENT WAVELENGTH OF LIGHT on fiber. - Essentially FDM for optical fiber. - DWDM (Dense WDM): 80–160 channels, spacing 0.8 nm or less. - Each lambda can carry 10–100 Gbps; total fiber capacity: multiple Tbps. - Example: Internet backbone (transcontinental fiber cables), Cisco's DWDM products. - Pros: Enormous capacity; transparent to data rate and format. 🔗 ANALOGY: FDM = multi-lane highway (each car in its own lane, all moving simultaneously). TDM = one-lane road with traffic signals (each car gets the full lane, but only briefly in turn). WDM = highway with lanes for different colors of cars (physical separation by wavelength). |  |

# 5. Guided & Unguided Transmission Media

| Q1: Compare twisted-pair, coaxial, and fiber optic cables. When would you use each? |  |
| --- | --- |
| ✅ Answer: - Twisted-Pair Cable (UTP/STP): - Pairs of copper wires twisted together to reduce crosstalk/EMI. - UTP (Unshielded): Cat5e (100 Mbps, 100m), Cat6 (1 Gbps, 100m), Cat6a (10 Gbps, 100m). - STP (Shielded): Better noise immunity, used in industrial environments. - Pros: Cheap, flexible, easy to install. Most common LAN cabling. - Cons: Limited distance (~100m for Ethernet), susceptible to noise. - Use: Office LAN, telephone lines, DSL. - Coaxial Cable: - Central conductor + insulator + mesh shield + outer jacket. - Thick-net (10Base5): 500m, 10 Mbps — obsolete. - Thin-net (10Base2): 185m, 10 Mbps — obsolete. - Current use: Cable TV (CATV), cable internet (DOCSIS), CCTV. - Pros: Better noise immunity than UTP, higher bandwidth. - Cons: Expensive, inflexible, difficult to install. - Fiber Optic Cable: - Light pulses through glass/plastic core. - Single-mode fiber (SMF): Core 8-10 μm, one light path, long distances (up to 100 km), used for WAN backbone. - Multi-mode fiber (MMF): Core 50-62.5 μm, multiple light paths, shorter distance (up to 2 km), used in data centers. - Speed: 10 Gbps to multiple Tbps with WDM. - Pros: No EMI, no signal loss, secure (hard to tap), enormous bandwidth. - Cons: Expensive, fragile, specialized installation. - Use: Internet backbone, data centers, submarine cables, FTTH (Fiber to Home). 💡 TIP: Cisco SDE question: 'What cable runs from access switch to desktop?' → Cat6 UTP. 'What connects data centers?' → SMF fiber. 'What cable does your ISP use for backbone?' → SMF with DWDM. |  |
| Q2: Explain radio waves, microwaves, and infrared as unguided media. |  |
| ✅ Answer: - Radio Waves (3 Hz – 1 GHz): - Omnidirectional; no line of sight needed. - Penetrate walls and buildings. - Examples: AM/FM radio, Wi-Fi (2.4 GHz technically microwave but colloquially radio), Bluetooth. - Issue: Susceptible to interference; regulated by government (FCC, TRAI). - Microwaves (1 GHz – 300 GHz): - Unidirectional; requires LINE OF SIGHT between antennas. - Cannot penetrate buildings; requires careful antenna alignment. - Types: Terrestrial (point-to-point, max ~50 km) and Satellite. - Examples: Wi-Fi (2.4 & 5 GHz), cellular (3G/4G/5G), satellite TV, radar. - Satellite: GEO (35,786 km, 270 ms latency), MEO, LEO (500-2000 km, <20 ms — e.g., Starlink). - Infrared (300 GHz – 400 THz): - Very short range; cannot penetrate obstacles. - Directional; requires line of sight. - Examples: TV remote controls, short-range device-to-device (old IrDA port on laptops). - Cannot be used for outdoor communication (blocked by fog, walls). - Security advantage: Does not penetrate walls → contained within room. 🔗 ANALOGY: Radio waves = shouting in all directions. Microwaves = flashlight (focused beam, needs clear path). Infrared = laser pointer (very directional, very short range). |  |

# 6. Data Link Layer — Error Detection, Framing & Flow Control

| Q1: Distinguish between single-bit errors and burst errors. Which is more common and why? |  |
| --- | --- |
| ✅ Answer: - Single-bit Error: - Only ONE bit in a data unit is changed (0→1 or 1→0). - Very rare in practice — requires noise affecting exactly one bit. - More common in parallel transmission (where each bit uses separate line). - Burst Error: - TWO OR MORE bits in a data unit are changed. - Length = from first corrupted bit to last corrupted bit (including uncorrupted bits in between). - Common in serial transmission (single channel) where noise affects a continuous period. - Example: A 1000 bps link with 1 ms noise burst → 1 bit corrupted. But a 1 Gbps link with 1 ms noise burst → 1 million bits affected! Hence burst errors are critical at high speeds. Why burst errors are more common: - Real noise (lightning, motor interference) affects signal for a DURATION, not a single bit. - During that duration, multiple bits pass and get corrupted. - Modern fiber/copper transmission is serial → burst errors dominate. 💡 TIP: At Cisco interview: 'Why do CRC codes protect against burst errors better than simple parity?' Because CRC detects ALL burst errors up to the degree of the polynomial (e.g., CRC-32 catches all burst errors ≤32 bits). |  |
| Q2: Explain Hamming Code. How do you calculate the number of redundancy bits needed and place them? |  |
| ✅ Answer: Hamming code provides SINGLE-BIT ERROR CORRECTION (and 2-bit error detection with an extra parity bit). Number of redundancy bits (r): Condition: 2^r ≥ m + r + 1, where m = data bits, r = redundancy bits. Example: m = 7 data bits → 2^4 = 16 ≥ 7+4+1 = 12 ✓ → r = 4 redundancy bits. So we need 11 total bits (7 data + 4 parity). Parity bit positions: Powers of 2 → positions 1, 2, 4, 8, 16... How they work: - P1 (pos 1): checks bits 1,3,5,7,9,11... (odd positions) - P2 (pos 2): checks bits 2,3,6,7,10,11... - P4 (pos 4): checks bits 4,5,6,7,12,13... - P8 (pos 8): checks bits 8,9,10,11,12... To find error position: Compute syndromes (S1 from P1, S2 from P2, S4 from P4...). Error position = S4×4 + S2×2 + S1×1. Flip that bit! If all syndromes are 0 → no error. If syndromes indicate non-zero position → flip that bit to correct. Extended Hamming (SECDED): Add one more overall parity bit → detect 2-bit errors (though cannot correct them). 🔗 ANALOGY: Hamming code is like a 3D coordinate system — each parity bit covers a different 'axis', and their combination pinpoints the exact location of the error in 3D space. |  |
| Q3: Explain CRC (Cyclic Redundancy Check). How is the divisor chosen and what errors does it detect? |  |
| ✅ Answer: CRC uses polynomial division to detect errors. How it works: 1. Sender appends r zero bits to data (where r = degree of generator polynomial). 2. Divide augmented data by generator polynomial using XOR (modulo-2 arithmetic). 3. Remainder = CRC (also called FCS — Frame Check Sequence). 4. Replace the r zeros with this remainder; transmit. 5. Receiver divides received frame by same generator; if remainder = 0, no error. Example: Data = 1001, Generator = 1011 (degree 3, so r=3). Augmented data = 1001000. Divide 1001000 by 1011 using XOR. Result: quotient (discarded) + remainder. Append remainder to data. Transmit. Standard generators: - CRC-16 (x^16+x^15+x^2+1): Detects all single and double bit errors, all odd-error-count errors, all burst errors ≤16 bits. - CRC-32 (used in Ethernet, ZIP): Detects all burst errors ≤32 bits. Probability of missing error = 1/2^32. - CRC-CCITT: Used in HDLC, PPP protocols. What CRC detects: - All single-bit errors (always). - All double-bit errors (with proper polynomial). - All odd number of bit errors (if polynomial includes x+1 as factor). - All burst errors ≤ degree r. - Most burst errors > r (probability of missing = 1/2^r). CRC does NOT correct errors — it only detects them. 💡 TIP: Ethernet (IEEE 802.3) uses CRC-32. Every Cisco switch and NIC performs CRC on every frame. If CRC fails, the frame is silently dropped. That's why TCP retransmits — it's the transport layer that handles retransmission. |  |
| Q4: Explain ALOHA, Slotted ALOHA, CSMA, CSMA/CD, and CSMA/CA. Compare their efficiency. |  |
| ✅ Answer: These are Multiple Access Protocols for shared media: - Pure ALOHA: - Transmit whenever you want. If collision → wait random time → retransmit. - Efficiency: max 18.4% (1/(2e)). - Vulnerable time = 2T (frame transmission time). - Slotted ALOHA: - Divide time into slots. Transmit only at beginning of a slot. - Halves the vulnerable time → doubles efficiency. - Efficiency: max 36.8% (1/e). - Used in: Early satellite channels, RFID readers. - CSMA (Carrier Sense Multiple Access): - 'Listen before you talk.' Check if channel is idle before transmitting. - Still collisions possible (propagation delay means two stations may both sense idle simultaneously). - Reduces but doesn't eliminate collisions. - CSMA/CD (Collision Detection): - Sense before transmit + detect collision during transmission + stop + jam signal + random backoff (BEB — Binary Exponential Backoff). - Used in: Wired Ethernet (IEEE 802.3) with shared medium (hub-based). - Not used with switches (switches create separate collision domains — no need for CD). - Minimum frame size = 2 × propagation delay × bandwidth (ensures collision is detected). - Ethernet minimum frame = 64 bytes. - CSMA/CA (Collision Avoidance): - Used in wireless (IEEE 802.11 Wi-Fi) where collision detection is impossible. - Cannot detect own collision (hidden node problem, signal propagation differences). - Uses: IFS (Interframe Space) gaps + random backoff before transmitting + ACK. - DCF (Distributed Coordination Function) is the basic mode. - RTS/CTS (Request To Send / Clear To Send) solves hidden node problem. 📝 Note: Cisco switches eliminate collision domains — each port is its own collision domain. So CSMA/CD is irrelevant on switched Ethernet. CSMA/CA is critical for Cisco Wi-Fi (Aironet/Catalyst wireless). 🔗 ANALOGY: ALOHA = everyone talks at once in a room. CSMA = listen first, then speak. CSMA/CD = speak, hear collision, stop immediately. CSMA/CA = raise hand, wait for clearance before speaking (wireless classroom protocol). |  |
| Q5: Explain Sliding Window Protocols: Go-Back-N vs. Selective Repeat. What are the window size limitations? |  |
| ✅ Answer: Sliding window protocols allow multiple frames 'in flight' simultaneously, improving throughput. - Go-Back-N (GBN): - Sender window size: up to 2^n - 1 (where n = bits in sequence number). - Receiver window size: 1 (receiver accepts ONLY in-order frames). - On error at frame k: Retransmit frame k AND ALL SUBSEQUENT frames (even if correctly received). - Simple receiver, wasteful retransmission. - Example: n=3 bits → sender window max = 7 frames in flight. - Selective Repeat (SR): - Sender AND receiver window size: up to 2^(n-1). - On error at frame k: Retransmit ONLY frame k. - Receiver buffers out-of-order frames, delivers in-order to upper layer. - Efficient but complex receiver (needs buffer). - Example: n=3 bits → window max = 4 frames each side. Why window size matters: - GBN: Window < 2^n (not 2^n) to avoid sequence number ambiguity. - SR: Window ≤ 2^(n-1) strictly to avoid confusion between new and retransmitted frames. - Window size must match the bandwidth-delay product for maximum efficiency. Efficiency: - If error rate is low → SR and GBN perform similarly. - If error rate is high → SR is much better (less retransmission). 💡 TIP: Interview trick: 'What's the max window size for 3-bit sequence numbers?' GBN = 7, SR = 4. Confusing them is a common mistake. Memorize: SR window = half of 2^n. |  |

# 7. Network Layer — IP Addressing & Routing

| Q1: Explain classful IPv4 addressing. What are the classes, their ranges, and limitations? |  |
| --- | --- |
| ✅ Answer: IPv4 = 32-bit address (4 billion addresses total). Class A: 0.0.0.0 – 127.255.255.255 ¦ First bit: 0 ¦ Network: 8 bits, Host: 24 bits ¦ 2^7=128 networks, 2^24-2 ≈16M hosts each ¦ Used by: Large orgs, ISPs. Class B: 128.0.0.0 – 191.255.255.255 ¦ First bits: 10 ¦ Network: 16 bits, Host: 16 bits ¦ 2^14=16K networks, 65K hosts each ¦ Used by: Universities, large companies. Class C: 192.0.0.0 – 223.255.255.255 ¦ First bits: 110 ¦ Network: 24 bits, Host: 8 bits ¦ 2^21=2M networks, 254 hosts each ¦ Most common. Class D: 224.0.0.0 – 239.255.255.255 ¦ First bits: 1110 ¦ Multicast groups. Not assigned to hosts. Class E: 240.0.0.0 – 255.255.255.255 ¦ First bits: 1111 ¦ Reserved for research. Private IP Ranges (RFC 1918): - Class A: 10.0.0.0/8 - Class B: 172.16.0.0/12 (172.16 to 172.31) - Class C: 192.168.0.0/16 Limitations of Classful Addressing: - Rigid class sizes lead to wasted addresses (Class B gave 65K hosts — most didn't need that many). - Rapid exhaustion of IPv4 space. - No flexibility to give a company exactly what they need. - Solution: CIDR (Classless Inter-Domain Routing). 💡 TIP: Quick recall: 'A=0, B=10, C=110, D=1110, E=1111' — the first bits tell you the class. Class A starts with 0, so range is 0-127 (01111111 = 127). |  |
| Q2: Explain CIDR (Classless Addressing) and subnetting with a worked example. |  |
| ✅ Answer: CIDR (Classless Inter-Domain Routing): Allows variable-length subnet masks (VLSM) — not restricted to class boundaries. Notation: IP/prefix (e.g., 192.168.1.0/24 means first 24 bits are network, last 8 are host). Subnetting Example: Given: 192.168.1.0/24. Divide into 4 subnets. - Need 4 subnets → borrow 2 bits from host (2^2 = 4). - New prefix: /26 (24 + 2 = 26). - Each subnet has 2^6 = 64 addresses (62 usable hosts — subtract network and broadcast). Subnets: - 192.168.1.0/26: hosts .1 to .62, broadcast .63 - 192.168.1.64/26: hosts .65 to .126, broadcast .127 - 192.168.1.128/26: hosts .129 to .190, broadcast .191 - 192.168.1.192/26: hosts .193 to .254, broadcast .255 Important formulas: - Number of subnets = 2^(borrowed bits) - Hosts per subnet = 2^(host bits) - 2 Supernetting (Route Aggregation): - Combine multiple networks into one larger one. - Example: 192.168.0.0/24 + 192.168.1.0/24 + 192.168.2.0/24 + 192.168.3.0/24 → 192.168.0.0/22. - Reduces routing table size (route summarization) — critical for scalable internet routing. 💡 TIP: VLSM subnetting is tested heavily at Cisco. Practice: Given a block, allocate subnets to departments of different sizes efficiently (largest first). This is a core network engineer skill. |  |
| Q3: What is NAT? Why is it used? Explain Static NAT, Dynamic NAT, and PAT (NAPT). |  |
| ✅ Answer: NAT (Network Address Translation): Translates private IP addresses to public IP addresses (and vice versa) at a router/firewall. Why NAT: - IPv4 address exhaustion (only ~4 billion public IPs for 8 billion people + IoT). - Security: Internal hosts hidden behind single public IP. - Flexibility: Change ISP without renumbering internal network. Types of NAT: - Static NAT: One-to-one permanent mapping. Each private IP maps to a specific public IP. - Use case: Hosting a server inside (web server, mail server) — always reachable at same public IP. - Dynamic NAT: Many-to-many from a pool. Private IPs mapped to available public IPs from a pool. - When pool exhausted → new connections dropped. - Use case: Organization with multiple public IPs, no need for permanent mapping. - PAT (Port Address Translation) / NAPT / NAT Overload: - Many-to-one. ALL private IPs share a SINGLE public IP, differentiated by PORT NUMBER. - Most common type (what home routers use). - Maps (private IP:private port) → (public IP:public port). - Router maintains state table; incoming packets matched to correct internal host. - Example: Your home has 1 public IP from ISP; 5 devices share it via PAT. NAT limitations: - Breaks end-to-end connectivity (problematic for P2P, VoIP, gaming — need NAT traversal/hole punching). - Stateful — router must maintain translation table. - IPv6 eliminates need for NAT (128-bit addresses = 3.4 × 10^38 addresses). 🔗 ANALOGY: PAT = office building with one main phone number (public IP) and internal extensions (ports). Calls come in to main number and receptionist routes to the right extension. |  |
| Q4: Explain IPv6 addressing. What are its advantages over IPv4? |  |
| ✅ Answer: IPv6 = 128-bit address space = 3.4 × 10^38 unique addresses. Written as 8 groups of 4 hex digits: 2001:0DB8:85A3:0000:0000:8A2E:0370:7334. Abbreviation rules: - Leading zeros in each group can be omitted: 0DB8 → DB8. - Consecutive all-zero groups replaced with :: (only once in address). - 2001:0DB8:0000:0000:0000:0000:0000:0001 → 2001:DB8::1. IPv6 Address Types: - Unicast: One-to-one (Global Unicast: 2000::/3, Link-Local: FE80::/10, Loopback: ::1). - Multicast: One-to-many (FF00::/8). Replaces broadcast (no broadcast in IPv6!). - Anycast: One-to-nearest — same address assigned to multiple interfaces; packet goes to geographically nearest. IPv6 Advantages over IPv4: - Massive address space (eliminates NAT requirement). - Simplified header (fixed 40-byte header vs. variable IPv4 header). - No fragmentation at routers (only at source — Path MTU Discovery). - Built-in IPsec support (mandatory in IPv6 spec). - Auto-configuration (SLAAC — Stateless Address Auto-Configuration, no DHCP needed). - No broadcast (uses multicast — more efficient). - Flow label field for QoS. - Hierarchical addressing for efficient routing. 💡 TIP: Key exam fact: IPv6 header is FIXED at 40 bytes (vs. IPv4 20-60 bytes). IPv6 has NO checksum in header (left to transport layer). NO fragmentation by routers. These make IPv6 router processing FASTER. |  |
| Q5: Compare Distance Vector Routing (DVR) and Link State Routing (LSR). Which protocols use each? |  |
| ✅ Answer: - Distance Vector Routing (DVR): - Each router knows only distance to destinations and which neighbor to use (next hop). - Routers share their full routing tables only with DIRECT NEIGHBORS periodically. - Algorithm: Bellman-Ford equation: D(x,y) = min over v {cost(x,v) + D(v,y)} - Slow convergence — good news travels fast, bad news travels slow (count-to-infinity problem). - Count-to-infinity problem: Link failure can cause routing loops; increments hop count indefinitely. - Solutions: Split horizon, Poison reverse, Hold-down timers, Max hop count (RIP: 15 hops max). - Protocols: RIP (Routing Information Protocol), IGRP (Cisco proprietary). - Pros: Simple, low CPU/memory. - Cons: Slow convergence, count-to-infinity, doesn't scale well. - Link State Routing (LSR): - Each router knows the COMPLETE network topology. - Routers flood LSAs (Link State Advertisements) to ALL routers. - Each router runs Dijkstra's SPF algorithm to compute shortest path to all destinations. - Fast convergence — every router has complete map. - No count-to-infinity (complete topology known). - Protocols: OSPF (Open Shortest Path First), IS-IS (Intermediate System to IS). - Pros: Fast convergence, scalable, loop-free, supports hierarchical design (areas in OSPF). - Cons: More CPU/memory (Dijkstra computation), more bandwidth during flooding. Comparison Table: - Basis of knowledge: DVR=partial (neighbors) ¦ LSR=full (all routers). - Algorithm: DVR=Bellman-Ford ¦ LSR=Dijkstra. - Info shared with: DVR=neighbors only ¦ LSR=all routers (flooding). - Convergence: DVR=slow ¦ LSR=fast. - Protocol examples: DVR=RIP ¦ LSR=OSPF, IS-IS. 📝 Note: Cisco enterprise networks use OSPF (link state) internally. BGP is a path-vector protocol (hybrid, used for Internet routing between ISPs). Cisco CCNA/CCNP heavily covers OSPF. 🔗 ANALOGY: DVR = asking your neighbor for directions (they only know their local area). LSR = having a complete GPS map of the entire city. |  |

# 8. Transport Layer — TCP, UDP & Protocols

| Q1: Compare TCP and UDP in detail. When would you choose UDP over TCP? |  |
| --- | --- |
| ✅ Answer: TCP (Transmission Control Protocol): - Connection-oriented: 3-way handshake (SYN, SYN-ACK, ACK) before data. - Reliable: Guarantees delivery, ordered delivery, no duplicates. - Error control: Checksum + ACK + retransmission. - Flow control: Sliding window (receiver advertises window size). - Congestion control: Slow start, AIMD (Additive Increase Multiplicative Decrease). - Full-duplex: Simultaneous bidirectional data. - Header: 20-60 bytes (variable due to options). - Use cases: HTTP/HTTPS, FTP, SMTP, SSH, database connections. UDP (User Datagram Protocol): - Connectionless: No handshake; just send datagrams. - Unreliable: No guarantee of delivery, order, or duplicate-free. - No flow/congestion control. - Header: 8 bytes (Source Port, Dest Port, Length, Checksum only). - Very low overhead — fast. - Use cases: DNS, DHCP, SNMP, VoIP, video streaming, online gaming, IoT. When to choose UDP: - Speed is critical, small loss acceptable (VoIP: a dropped packet is better than delayed one). - Application implements its own reliability (QUIC/HTTP3 over UDP). - Broadcast/multicast (TCP can't multicast). - Simple request-response (DNS: one packet out, one packet back — no need for connection setup). - Real-time: Gaming, live video — old data is useless; new data is more valuable. 💡 TIP: Cisco IOS/routers use UDP for SNMP (port 161/162), TFTP (port 69), and NTP (port 123). TCP is used for BGP (port 179), HTTPS management, SSH. Knowing the port numbers shows interviewer you've worked in real networks. |  |
| Q2: Explain TCP's 3-way handshake and 4-way teardown. What happens if a SYN packet is dropped? |  |
| ✅ Answer: 3-Way Handshake (Connection Establishment): Step 1 - SYN: Client sends segment with SYN flag, ISN (Initial Sequence Number) = x. Step 2 - SYN-ACK: Server responds with SYN flag + ACK flag. Server's ISN = y. ACK = x+1. Step 3 - ACK: Client sends ACK = y+1. Connection established. 4-Way Teardown (Connection Termination): Step 1 - FIN: Client sends FIN (finished sending). Client enters FIN_WAIT_1. Step 2 - ACK: Server acknowledges FIN. Client enters FIN_WAIT_2. Server may still send data. Step 3 - FIN: Server sends its own FIN when done. Step 4 - ACK: Client sends final ACK. Client enters TIME_WAIT (2×MSL ≈ 60 seconds). TIME_WAIT: Why 2×MSL? - Ensures final ACK reaches server (if lost, server resends FIN). - Prevents old duplicate packets from a previous connection being accepted by new connection. If SYN packet is dropped: - Sender retransmits SYN after RTO (Retransmission Timeout). - RTO doubles each time (exponential backoff): 1s → 2s → 4s → 8s... - After several attempts (OS-dependent, typically 5-6), connection fails with ETIMEDOUT. - SYN flood attack exploits this: attacker sends many SYNs, server creates half-open connections, exhausting memory. Mitigated by SYN cookies. 🔗 ANALOGY: 3-way handshake = calling someone: you say 'Hello?' (SYN), they say 'Hello, I hear you!' (SYN-ACK), you say 'Great, I hear you too!' (ACK). Now conversation begins. |  |
| Q3: Explain Go-Back-N (GBN) vs Selective Repeat (SR) at the transport layer. How does TCP use them? |  |
| ✅ Answer: These are ARQ (Automatic Repeat reQuest) protocols for reliable data transfer: Stop-and-Wait: - Send ONE frame, wait for ACK before sending next. - Utilization = T_transmission / (T_transmission + 2 × T_propagation). - Very inefficient for large bandwidth-delay products. Go-Back-N (GBN): - Window size ≤ 2^n - 1. - On timeout or NAK: retransmit current frame AND all subsequent frames. - Receiver: only accepts frames in order; discards out-of-order frames (sends same ACK). - Simple receiver, poor efficiency under high error rates. Selective Repeat (SR): - Window size ≤ 2^(n-1). - On timeout: retransmit ONLY the specific lost/corrupted frame. - Receiver buffers out-of-order frames. - More efficient, more complex. How TCP uses these concepts: - TCP uses a form of Selective Repeat (SACK — Selective Acknowledgment, RFC 2018). - Without SACK: TCP behaves like Go-Back-N (resends from lost segment onward). - With SACK: Receiver tells sender exactly which segments arrived; sender resends only missing ones. - Modern TCP (with SACK) is much more efficient, especially on lossy links or satellite links. - SACK is negotiated during SYN exchange and is nearly universally supported today. 💡 TIP: In a Cisco interview: 'Does TCP use GBN or SR?' Best answer: 'TCP traditionally resembled GBN, but with the SACK option (RFC 2018) it uses Selective Repeat, significantly improving performance on lossy links.' |  |
| Q4: Explain TCP congestion control: Slow Start, Congestion Avoidance, Fast Retransmit, and Fast Recovery. |  |
| ✅ Answer: TCP congestion control prevents sender from overwhelming the network. Key variables: - CWND (Congestion Window): Sender-side limit. - RWND (Receiver Window): Receiver-side limit. - Effective window = min(CWND, RWND). - ssthresh (Slow Start Threshold): Divides slow start from congestion avoidance. Phases: 1. Slow Start: - Start CWND = 1 MSS (Maximum Segment Size). - Double CWND every RTT (exponential growth). - Until: CWND reaches ssthresh OR packet loss detected. - Note: Called 'slow' because starts at 1, not because growth is slow. 2. Congestion Avoidance (AIMD): - When CWND ≥ ssthresh. - Increase CWND by 1 MSS per RTT (linear growth — additive increase). - On loss: ssthresh = CWND/2; CWND = 1 (Tahoe) or CWND = ssthresh (Reno). 3. Fast Retransmit: - On receiving 3 duplicate ACKs → segment lost (don't wait for timeout). - Immediately retransmit the missing segment. - 3 dup-ACKs = mild congestion. Timeout = severe congestion. 4. Fast Recovery (TCP Reno/NewReno): - After fast retransmit: ssthresh = CWND/2, CWND = ssthresh + 3. - Stay in fast recovery (inflate CWND for each additional dup ACK). - On new ACK: exit fast recovery; CWND = ssthresh. - Avoids dropping to 1 MSS (more efficient than Tahoe). TCP CUBIC (default in Linux 3.2+): - Uses cubic function to grow CWND based on time since last loss. - Better performance on high-bandwidth, high-latency links (e.g., 10G networks). 🔗 ANALOGY: TCP congestion control = driving on a highway: start slow, accelerate smoothly, slam brakes if accident, then cautiously accelerate again. CUBIC is like an adaptive cruise control. |  |
| Q5: What is the difference between Constant Bit Rate (CBR) and Variable Bit Rate (VBR) traffic? How does each affect network design? |  |
| ✅ Answer: - CBR (Constant Bit Rate): - Traffic generated at FIXED, predictable rate. No bursts. - Example: Uncompressed voice (PCM G.711: 64 Kbps exactly), TDM circuits, old CD-quality audio. - Network needs: Guaranteed bandwidth reservation, fixed delay (circuit-like behavior). - Suitable for: Circuit switching, ATM CBR service class, TDM. - Challenge: Inefficient — reserves bandwidth even during silence. - VBR (Variable Bit Rate): - Traffic rate VARIES over time; bursty by nature. - Real-time VBR: Compressed voice/video with variable quality (MPEG, H.264). Bounded variance. - Non-real-time VBR: Web browsing, email, file transfer. Highly bursty. - Example: Netflix streaming (VBR: action scene = high bitrate, static scene = low bitrate). - Network needs: Statistical multiplexing, QoS for bounded delay, adequate buffer. Key traffic metrics: - Average Data Rate: Mean over long period. - Peak Data Rate: Maximum burst rate. - Maximum Burst Size (MBS): Max consecutive bits at peak rate. - Effective Bandwidth: Bandwidth needed to ensure acceptable performance = somewhere between average and peak (depends on buffer size and delay tolerance). Impact on design: - CBR: Use WFQ (Weighted Fair Queuing) or CBWFQ with guaranteed rate. - VBR-rt: Use LLQ (Low Latency Queuing) for voice/video; strict priority queue. - VBR-nrt: Best-effort works; WRED (Weighted Random Early Detection) for fairness. 💡 TIP: Cisco QoS design always asks: 'What type of traffic is this?' CBR voice gets LLQ strict priority. Video uses CBWFQ. Data uses best-effort or WFQ. This maps directly to Cisco's MQC (Modular QoS CLI). |  |
| Q6: Explain network congestion: causes, effects, and the major congestion control mechanisms. |  |
| ✅ Answer: Congestion: Too many packets in a network segment → queues fill up → delays increase → packets drop → throughput falls. Causes of Congestion: - Multiple high-speed input lines feeding a slow output line. - Sudden burst from many sources simultaneously. - Inadequate buffer space at routers. - Slow processor at router causing queuing delays. Effects (Congestion Collapse): - Initially: Throughput increases with load. - Near saturation: Throughput plateaus (Cliff). - Beyond capacity: Throughput DROPS SHARPLY while delay INCREASES EXPONENTIALLY. - Reason: Routers drop packets → TCP retransmits → MORE packets → more congestion → spiral. Congestion Control vs. Flow Control: - Flow Control: End-to-end control between sender and receiver (prevents RECEIVER from being overwhelmed). - Congestion Control: Global control of network resources (prevents NETWORK from being overwhelmed). Congestion Control Mechanisms: Open Loop (Prevention — before congestion): - Traffic shaping: Leaky bucket (force CBR output from bursty input) and Token bucket (allow bursts up to bucket size). - Admission control: Refuse new connections if network near capacity (used in ATM, MPLS-TE). Closed Loop (Reaction — after congestion): - ECN (Explicit Congestion Notification): Routers mark packets instead of dropping; receivers inform senders. - Choke packets: Router sends congestion signal to source. - TCP congestion control: Slow start, AIMD, fast retransmit (react to packet loss as congestion signal). - RED/WRED: Randomly drop packets BEFORE queue full to give TCP advance warning. - Back pressure: Router tells upstream neighbor to slow down. 🔗 ANALOGY: Congestion = traffic jam. Congestion control = traffic management: metering lights on highway (leaky bucket), variable speed limits (AIMD), and emergency broadcasts (ECN/choke packets). Flow control = your exit's capacity limit, regardless of highway. |  |

# Interview Strategy & Final Tips from a 20+ Year Cisco Hiring Manager

These are the traits that separate candidates who get offers from those who don't:

- DEPTH over BREADTH: Always go one level deeper. 'TCP uses sliding window' — then be ready to explain GBN vs SR, SACK, and window scaling.

- REAL EXAMPLES: Tie every concept to a real product or scenario. 'Cisco Catalyst switches eliminate collision domains, making CSMA/CD irrelevant in modern switched Ethernet.'

- FORMULAS WITH INTUITION: Know Nyquist and Shannon, but also know WHEN to use each and WHAT the result means practically.

- TRADE-OFFS: Every design decision has trade-offs. Star vs mesh, TCP vs UDP, FDM vs TDM. Always discuss pros, cons, and use cases.

- OSI LAYER THINKING: When discussing any protocol or issue, immediately identify which OSI layer(s) are involved. This is the universal debugging framework.

- CISCO CONTEXT: Know where Cisco products fit — IOS, Catalyst switches (L2/L3), ASR routers (L3), Firepower (L3-L7 firewall), Meraki (cloud-managed), Webex (UCaaS over UDP/RTP).

- SUBNETTING SPEED: Practice CIDR subnetting until you can do it mentally. Given x.x.x.x/y, instantly know: network address, first/last host, broadcast, number of hosts.

- CONGESTION INSIGHT: Cisco QoS is a core product area. Know CBR vs VBR, LLQ vs CBWFQ, WRED, and how they map to real traffic types (voice, video, data).